國立東華大學資訊工程系博士班資格考

計算機結構, 2006 Spring

- (30 pts)

L.D.      F2,20(R2)

L.D.      F4,30(R3)

MULT.D   F0, F4, F6

SUB.D    F8, F2, F4

DIV.D     F10, F0, F2

ADD.D    F2, F8, F4

(a) Identify instruction pairs with data dependence, anti-dependence and output dependence. (15 pts)

(b) How the Tomasulo algorithms resolve WAR and WAW hazards? (5 pts)

(c) List 5 hardware or software techniques to minimize data hazards in the processor? (10 pts)

- (15 pts) Suppose we have a deeply pipelined processor, for which we implement a branch-target buffer for the conditional branches only. Assume that the misprediction penalty is always 4 cycles and the buffer miss penalty is always 3 cycles. Assume 90% hit rate and 90% accuracy, and 15% branch frequency. How much faster is the processor with the branch-target buffer versus a processor that has a fixed 2-cycle branch penalty? Assume a base CPI without branch stalls of 1.

- (40 pts)Cache

- Explain the cache write policies “write through” and “write back”. (10 pts)

- Explain “write allocate” and “no-write allocate”. (10 pts)

- Explain three misses categories “compulsory”, “capacity”, and “conflict”. (10 pts)

- Describe one method to reduce cache miss penalty. (10 pts)

- (15 pts) What are the advantages of replicating data in multiple caches in the shared-memory multiprocessor? What problem will it introduce?